THE UNIVERSITY OF AZAD JAMMU AND KASHMIR

__________________________________________

| COURSE TITLE | CA&LD |
| --- | --- |
| COURSE CODE | CS-1205 |
| TITLE | Half Adder Implementation |
| STUDENT NAME | Tayyab Hanif |
| STUDENT ROLL NUMBER | 2024-SE-11 |
| INSTRUCTOR NAME | Engr. Sidra Rafique |
| Lab No: | 05 |

Lab 5: Half Adder Implementation

Objective

To design and verify the working of a Half Adder circuit using basic logic gates. The lab covers both the simple and extended (complex) half adder versions.

Apparatus Required

Breadboard

IC 7408 (AND Gate)

IC 7432 (OR Gate)

IC 7486 (XOR Gate)

IC 7404 (NOT Gate)

Power Supply (5V DC)

Connecting Wires

LEDs or Logic Probes

Theory

A Half Adder is a combinational logic circuit used to add two binary digits. It produces two outputs:

- Sum (S)

- Carry (C)

It adds two single-bit binary numbers A and B.

- Simple Half Adder

Boolean Expressions:

Sum (S) = A ⊕ B

Carry (C) = A · B

2. Complex (Extended) Half Adder

XOR Implementation using Basic Gates:

A ⊕ B = (A · ¬B) + (¬A · B)

So:

Sum (S) = (A · ¬B) + (¬A · B)

Carry (C) = A · B

Part A – Simple Half Adder

Circuit Description

Connect inputs A and B to a XOR gate to obtain the Sum output.

Connect inputs A and B to an AND gate to obtain the Carry output.

Truth Table – Simple Half Adder

| A | B | Sum (A ⊕ B) | Carry (A · B) |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 |

Observation

The Sum and Carry outputs match the expected results from the truth table. Thus, the simple half adder is verified.

Circuit Diagram

Implementation

Part B – Complex Half Adder Using Basic Gates

Boolean Expression for Sum

S = (A · ¬B) + (¬A · B)

Circuit Description

Connect input A to a NOT gate to get ¬A.

Connect input B to a NOT gate to get ¬B.

Use AND gates to get A · ¬B and ¬A · B.

Feed these two outputs into an OR gate to get the Sum.

Use an AND gate with inputs A and B to get the Carry.

Truth Table – Complex Half Adder

| A | B | ¬A | ¬B | A·¬B | ¬A·B | Sum | Carry |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 |
| 1 | 0 | 0 | 1 | 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |

Observation

The outputs of the complex half adder match the expected outputs. The XOR function is successfully implemented using only AND, OR, and NOT gates.

Circuit Diagram

Implementation

Conclusion

Both simple and complex versions of the half adder were implemented and tested successfully. The results confirm that:

XOR gates simplify the design of half adders.

XOR functionality can also be replicated using basic gates, validating the theoretical logic expressions.